Flight-Line Operations

Rhode Island Wing

Civil Air Patrol

Revised October 2003

This is a Rhode Island Wing-Civil Air Patrol document only.  While we have tried to adhere to all applicable National CAP and Federal Air Regulations in force as of the revision date, we have also greatly expanded the content in the interest of safety. The latest CAP regulations and Federal Air Regulations should always be followed and may change from time to time. For the latest regulations and information, please refer to web sites at Civil Air Patrol: <http://www.cap.gov> and the Federal Aviation Administration at: <http://www.faa.gov>.  As of August 2003, the specific material from the FAA was at:

<http://www1.faa.gov/ATpubs/AIM/Chap4/aim0403.html#4-3-25>

Expanded Air Force marshaling procedures, when needed, can be found in Air Force Instruction 11-218 which is presented on-line courtesy of California Wing, CAP at:

<http://cawg.cap.gov/Files/Manuals/AFI%2011-218.pdf>

CAP emblem copyright Civil Air Patrol.  All other text and graphics are considered public domain and may be freely distributed, adapted, and modified as desired. Please send a courtesy copy to us if improvements to safety were made.

Comments or suggested revisions on this document should be addressed to:

Rhode Island Wing, Civil Air Patrol

Attn: Air Operations

# 24 Melrose Street

West Warwick, RI  02893-4115

Original document and revisions of October 2003 authored by

Dr. John Concannon, Major, CAP

jconcannon@lifespan.org

- CAP Aircraft Marshals are responsible for providing standard marshaling signals, in a clear and precise manner, to arriving and departing CAP aircraft. Their actions help ensure safe flight operations on the ground by coordinating the movement, parking, and securing of mission aircraft.  These responsibilities are not to be taken lightly for the active flight-line area can be a dangerous place, and full attention must be paid to guard against both severe personal injury and costly aircraft damage.

- The Flight Line Supervisor has overall command and responsibility of all flight line and marshaling activities. Flight Line Marshalers are to report to the Flight Line Supervisor, who in turn, reports to the Air Operations Branch Director (commonly referred to as Air Ops).  Given the hazards involved, all also work closely with the Mission Safety Officer. Further information on command and reporting procedures is available in CAPR 60-3, and on Flight-Line Marshal certification in CAPR 60-4 Vol II.

- No one should attempt to marshal an aircraft unless trained and approved by a Flight Line Supervisor. Closely supervise cadets at all times.  Check all 101 and 101T cards for training.  CAP Flight Marshals may not operate contrary to FAA regulations or operate outside of the immediate area of the general aviation operations area (aka flight-line, GA ramp, apron, or tarmac) where they are marshaling aircraft in support of the mission.

- Remember that not all aircraft on the ramp are necessarily involved with the mission, so don't try to tell pilots where to park or how to operate unless you know they're CAP aircraft or they indicate they request your assistance; otherwise stay clear. There may be significant liability issues involved when marshaling airplanes (both CAP and non-CAP). Specific approval of the Wing Commander is required before participating in marshalling non-CAP aircraft at air shows, etc.

- Be aware that not all pilots are familiar with all of the standard flight line hand signals (civilian FBO marshalers often use nonstandard hand signals).  Ultimately, the pilot of the aircraft has the obligation and authority to do whatever they think is necessary for safe flight operations, even if it is contrary to the marshal instructions.  When in doubt, brief the pilot on hand signals that will be used prior to their boarding the aircraft.

- Attempt to establish security to keep unauthorized people away from CAP aviation operations.  Remember, you have no authority whatsoever over the non-CAP public (but by your manners and dress you can give them the impression that you do). Always show courtesy; you may be able to recruit a new CAP member! Report any security problems or potential threats to Air Ops and/or the airport police immediately.

- Frequencies for air to ground and for ground to ground communications should be set ahead of time by Air Ops.  Establish procedures beforehand to report aircraft departure and arrival times to Air Ops, if tasked to do so.  The Flight Line Commander should have immediate access to radios than can both monitor airport tower/ground frequencies, and communicate with Air Ops.

- Be sure that both the airport tower and the airport security have been advised in advance of your presence and plans on the airfield. The flight line supervisor at large airports (PVD) should be issued and wear a flight line ID badge at all times on the tarmac.

Diagram of typical general aviation ramp flight line. How many hazards can you spot?

- Coordinate with airport services (called a fixed base operator, or FBO for short) if necessary to choose appropriate parking slots on the ramp.  Ensure adequate clearance space wingtip to wingtip.  Painted tarmac parking markings usually only allow for small single engine aircraft space; a twin-engined or large wingspan aircraft will encroach into the adjoining spaces (see diagram).  Caution:  Most aircraft damage resulting from wing tip or rudder strikes occur when parking or repositioning airplanes.

- Check with the airport FBO ahead of time if avgas might be needed.  CAP may have a contract with the FBO.  If not, appropriate credit arrangements will need to be made through Air Ops.

- Flight line marshals should be wearing an orange/reflective vest at all times.  The flight line is a no hat area because hats may be blown off and sucked into aircraft engines causing considerable damage.  A hooded jacket, however, is permissible. Never attempt to chase after items blown into flight operations areas.

- Beware of jet or prop blast.  Eye protection is required at all times for marshalers on the flight line.  Ear protection/plugs must be used for extended operations and when appropriate.

- Police the ramp area for foreign objects and debris (FOD) before conducting flight line operations.  Dispose of FOD into appropriate trash containers. What goes around comes around; rocks and debris thrown into adjoining grass areas will later be re-thrown back into parked aircraft by grass mowers.

- Before commencing flight operations a flight line safety briefing is required and which should address all known hazards. Pay attention to safety at all times.  Therefore, the flight line is a no salute area.  Brief all flight line personnel on duties of wing walking, parking aircraft, securing aircraft, and control of personnel on ramp. Note the following partial list of hazards.

- Typical Flight-Line Hazard List

- Spinning propellers

- Non-spinning propellers

- Jet air intakes

- Taxiing aircraft

- Moving fuel and maintenance vehicles

- Exhaust fumes

- Aircraft engine noise

- Fuel chemical hazards

- Fuel explosion/fire

- Aircraft hangar doors opening

- Blind highways/taxiways at corners

- Slippery grease spots

- Ruts and holes in terrain

- Foreign objects and debris

- Windblown sand and dirt

- Aircraft wing/tail strikes

- Antennas

- Hot or cold aircraft surfaces

- Sunburn

- Heat exhaustion

- Dehydration

- Glare and snow blindness

- Ice, snow, and rain

- Wind chill and frostbite

- Lightning strikes

- Wild animals and insects

- Nightblindness

- Fatigue

- Hunger/thirst

- Inattention

- Disorientation

- Marshaling signal confusion

- Coordinate with the outgoing pilot any special needs or signals to be used while marshaling.  A thumbs up from the pilot will indicate readiness to taxi out.  Ensure chocks, etc, are removed prior to engine start.

- Never stand directly in front of a close-by operating aircraft.  Tricycle geared aircraft have less visibility than you might think while on the ground.  Tail-dragging aircraft have no forward visibility at all.  Always position yourself in view of the left seat pilot when marshaling a close-by airplane. Never allow persons or vehicles to pass between a marshaler and a running aircraft.

- One exception to the usual marshalers position is that if the setting sun is positioned directly into the pilots eyes in that position, move to a position off of the left wing and marshal from there. While CAP should not likely be marshaling helicopters, remember that, in this case, the pilot sits on the right side, so marshal from the right. Applicable helicopter marshaling signals and procedures can be found in AFI 11-218 as referenced on Page 1 of this document.

- Immediately prior to engine start, all aircraft will (usually) turn on their aircraft beacon (blinking red light on top of tail or fuselage). The pilot of a small airplane will open the aircraft window and yell out "Clear!", just before engine start.  Pilots of larger aircraft and jet aircraft do not.  In either event, beware that an aircraft engine could start up at any time.  Always treat propellers as if they were turning. Stay well clear of the propeller or intake areas at all times.  Hand propping of engines is never allowed, even of ordered to do so by a pilot or a superior officer.

- Assign a fire guard who has been properly trained.   All flight line personnel should be briefed on proper operation of the extinguisher if needed.  Halon extinguishers (available at Quonset) are expensive, hazardous, and not to be played with, but are the best for engine-start fires. Aircraft fire evacuation drills should occur at least once during each practice mission.  Coordinate with the pilot (only) on this ahead of time. For obvious reasons, there is no smoking on the flight line.

- Small engine fires may occur at engine start.  In a small piston powered airplane, the pilot may elect to attempt to extinguish the fire by continuing to crank the engine.  Do not use a fire extinguisher in this case unless instructed to do so by the pilot.

Once the engine has started successfully allow several minutes for the aircrew to perform necessary radio checks and to obtain clearances from the tower. The marshal should stand by at parade rest until the pilot gives a "Thumbs Up" to indicate readiness to taxi. Be sure to direct the aircraft in the appropriate direction for the taxiway on their way to the run-up area (a designated location on the airport taxiway for aircraft run up the engine and test all systems prior to flight).

- Upon completion of marshaling an outbound aircraft, snap to attention and give a quick hand salute to the pilot.  This salute should be returned, but may not be visible to the marshaler.  Report wheels off time to Air Ops.

- When marshaling an inbound aircraft, monitor the published tower, unicom, or ground frequency as appropriate.  Report aircraft arrival and wheels down time to Air Ops. If operating from an airport with an operational control tower, the control tower will give pilots taxi instructions by radio, until off the taxiways.

- Begin marshaling the inbound aircraft once it has turned off the active runway to the taxiway leading to the GA ramp area.  If the aircraft has an unobstructed view of the marshaler, stand at where you want the nose of the aircraft to park and raise your hands full up to signal the pilot ‘This Marshaler’.  If the view to the parking location is obstructed by other aircraft, the marshaler should proceed safely out far enough onto the tarmac to be seen by the taxiing pilot.  Once visual contact is ascertained, slowly walk to the spot where the nose should be parked, maintaining eye to eye contact with the pilot at all times.  Once the aircraft is aligned with the desired parking spot and approaches, the marshaler should step back and to the left 25 feet while maintaining visual contact with the pilot.  The marshaler should quickly look to the rear periodically while walking to ensure their own safety. Ensure adequate space between adjoining aircraft as the plane taxis into the parking slot.  Attempt to align all aircraft nose tips into a neat row along the tarmac.

- Do not approach closer than 25 feet to the aircraft until the prop has completely stopped spinning and instructed to do so by the pilot, or when the door opens. Never chock a propeller aircraft until the door is opened or signaled to do so by the pilot.  Expensive jet aircraft may require chocking of nose gear before engine shutdown, but exercise extreme caution to avoid intake areas.

- After the aircrew has exited the aircraft, welcome them to the base, and direct them to the location of Air Ops.  Be on guard for wobbly legs of aircrew who have been sitting in cramped quarters for hours on end.  It helps to know where the nearest bathrooms are.  Arrange for fuel if requested to do so by the pilot.  Ensure that the proper grade of fuel is used (usually 100LL in CAP aircraft), static ground lines are used, and that the correct amount of fuel requested is transferred. Submit any aircraft fuel receipts to Air Ops.

- Ensure wheel chocks and tie-downs are on hand, and use them when necessary.  Do not use chocks if freezing rain or snow is predicted.  Only aircrew should remove chocks, before start-up, as part of their routine pre-flight check. Turbine aircraft may require removal of chocks after start-up, but this should be signaled by the pilot with a thumbs outward motion, and done only with extreme caution for the hazards of the operating engine.

- Clean aircraft windows as necessary with approved aircraft plexiglass cleaner only.  Never use any other window cleaners such as Windex; crazing may result.

- Do not otherwise touch aircraft unless specifically instructed to do so by the pilot or Air Ops. Tie downs, wheel chocks, and gust locks are ultimately the responsibility of the pilot.  Any aircraft to be pushed into a new position should have a CAP pilot present during movement.

- For night operations preferably use a pair of luminescent cone-covered flashlights.  Never shine a flashlight directly at the pilot as it will destroy the night vision.

- On radiologic, chemical, or biological monitoring missions, mark off a decontamination area. Report any unsafe taxiing or aircraft problems to Air Ops.

- It is unlikely that Rhode Island CAP would be using vehicles in the airport operations area.  If CAP vehicles are necessary, they should be marshaled in the same way as aircraft when within fifty feet of an airplane.  CAP drivers should be reminded that at all times on the airport they should

- Drive with the driver's side toward aircraft

- Observe speed limit of 5 mph when within 50 feet of aircraft, otherwise 15 mph.

- Always operate with rotating amber beacon on while on the airport operations area

- Aircraft always have the right of way

- When approaching aircraft at night, turn off headlights to amber parking lights only.

- Be especially observant against strikes to aircraft wings and tails.

- If operating at an unimproved field (authorized only in actual missions if necessary):

- Survey air field for hazards.

- Advise Air Ops on field status.

- Prominently mark any found hazards.

- Survey the air field for best parking and taxi routes.

- Mark taxi routes and parking areas for mission aircraft as necessary.

- Map taxi routes and parking areas for briefing officer.

- Arrange for a "follow-me" vehicle if necessary and possible.

Flight-Line Equipment List

- Adequate weather protection:  Gloves & rain proof coat with integral hood.

- Orange/reflective vests for each person on the flight line marshaling team

- Ear plugs or ear cover headsets for marshals

- Eye protection:  goggles, eyeglasses, or sunglasses, preferably UV filtering

- Eye wash solution (May use saline contact lens solution) --for removal of foreign bodies blown into eyes.

- Sun protection (SPF-15 or greater) & insect protection (30% DEET or better)

Hand wands (batons) for each marshaler are preferred.  These can be dayglo colors for day use, or flashlights with wand attachments for nighttime

RI Airport Frequencies:

Airport		ID	Tower/CTAF

TFGreen	PVD	120.7

- PVD Ground     121.9

- NorthStar FBO  129.8

Newport	UUU	122.8

North Central 	SFZ	123.075

Quonset	OQU	126.35

- OQU Ground    134.5

Richmond	RIØ4	122.8

Westerly	WST	123.0

- Filled and operating fire extinguisher(s)

- Binoculars (to identify incoming CAP aircraft)

- Approved plexiglass cleaner & clean, nonabrasive cloth

- 2 way radio for comm with aircraft and air ops

- Air Ops freq:		_________________

- Mission Base freq:	_________________

- Air to Ground freq:	_________________

- Tower/CTAF freq:	_________________	(monitor)

- Ground freq:		_________________	(monitor)

- Multicom/FBO freq:	_________________	(monitor)

- Airport Police (RIAC): 	737-4000_________

- Airport Police (Outside RI): 1-866-427-3287

Notes: _______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

_______________________________________________________________________________

ALL CLEAR

or END OF

MARSHALING

(outbound)

CUT ENGINE

(Cold/Rain

Gear)

THIS

MARSHALER

SLOW

DOWN

START

ENGINE

FIRE!

Point at Source

NEXT

MARSHALER

or CLEARED TO TAXI

NIGHT

MARSHAL

Uses Same

Hand Signals

INSERTING

CHOCKS

SLOW STOP

Move Arms Gradually Up Until Crossed at Stop

MARSHALER

POSITION

AIRCRAFT MARSHALER HAND SIGNALS

TAXI

FORWARD

LEFT

TURN

RIGHT

TURN

EMERGENCY

STOP

Wave Arms to Cross